| Engine - Identification data |  |
| --- | --- |

If you have the engine tag still attached to the engine - just in front/below the thermostat housing

or perhaps the original engine number in the log book then the following should help you determine which engine you have.

Original engine identification numbers

850cc

| 8A | Austin up to 25000 |
| --- | --- |
| 8MB | Morris up to 25000 |
| 8AM | Austin & Morris 25000 onwards |
| 8AH | Austin & Morris Automatic |
| 8AJ | Austin & Morris closed circuit breathing |
| 8AK | Austin & Morris automatic with closed circuit breathing |
| 8WR | Wolseley Hornet & Riley Elf |
| 8AC | Moke |
| 85H/101 | All variants 1969 onwards |

Note: third suffix letter denotes compression type, L = Low, H = High, e.g. 8AM/U/H101 denotes high compression.

998cc

| 9WR | Wolseley Hornet & Riley Elf Mk2, pre closed circuit breathing |
| --- | --- |
| 9AD | Austin, Wolseley Hornet & Riley Elf Mk2 with remote type gearbox and closed circuit breathing |
| 9AE | Wolseley Hornet & Riley Elf Mk3 |
| 99H/-/101 on | Wolseley, Riley, Mini 1000, Clubman1000 1970 onwards and Mk3 with dished piston |
| 99H/791 | Mini 1974 onwards with dished piston |
| 99H/997 | 1980 onwards A+ block, flat top pistons, economy fitment with 2.95 final drive |
| 99H/A97P | A+ block with dished pistons, pre-A+ gears, 3.44 final drive & 12-inch wheels |

998 A+ block with centre locating tang in main cap, 1985 onwards

Flat top piston with circlip wrist pin retention

| 99H/B81 | Up to engine number 127431 |
| --- | --- |
| 99H/C20 | Up to engine number 105023 |
| 99H/997 | From engine number 127422 with 2.95 final drive |
| 99H/A35P |  |

Flat top piston with press fit wrist pin

| 99H/B81 | From engine number 127432 |
| --- | --- |
| 99H/C20 | From engine number 105024 |
| 99H/D80 |  |

Flat top pistons with press fit wrist pins, Lead-free cylinder head

| 99H/G30 or G32 or G33 |  |
| --- | --- |
| 99H/F15 or F16 |  |
| 99H/E20 or E21 |  |
| 99H/D81 |  |
| LBB10089 |  |

Dished pistons with circlip wrist pin retention

| 99H/B83P | Up to engine number 102908 |
| --- | --- |
| 99H/B84P | Up to engine number 100216 |

Dished pistons with press fit wrist pin

| 99H/B83P | From engine number 102909 |
| --- | --- |
| 99H/B84P | From engine number 100217 |

Dished pistons with press fit wrist pin, Lead-free

| 99H/G31 |  |
| --- | --- |
| 99H/E22 |  |
| LBB10175 |  |

1100cc (1098cc)

| 10AMW/Ta | Clubman Estate and Austin 110 (ADO16 shape) |
| --- | --- |
| 10H791 | Clubman saloon |
| 10H/-/H | All Austin 1100 2 & 4 door derivative Saloons (ADO16 shape) |
| 10GR/Ta/H | MG1100 (ADO16 shape) |
| 10GRB/Ta/H | MG1100 & Wolseley 1100 (ADO16) |
| 10V/Ta/H | Vanden Plas 1100 (ADO16 shape) |

997 Cooper

9F/Sa/H101 to 2637 except 19201 to 20410 (became 1070S)

998 Cooper January 1964 onwards)

| 9FA/Sa/H | Mk1 Austin pre closed circuit breathing |
| --- | --- |
| 9FD/Sa/H | Mk1 Morris up to engine number 1934, all Austin Mk2 closed circuit breathing and Morris Mk2 from engine number 1935-33660 with closed circuit breathing |
| 9FD/Xe/H | 4 syncro gearbox introduced |
| 99/-/H | Last of manufacture 1969 |

Cooper S variants

970cc

| 9F/Sa/X | Engine numbers 29001-29003 with Tecalamit filter |
| --- | --- |
| 9FD/Sa/X | Closed circuit breathing introduced with Tecalamit filter |
| 9FE/Sa/X | Closed circuit breathing, oil filter with oil light switch introduced |
| 9F/Sa/X | Engine numbers 29039-30029 Purolator oil filter with oil light switch and closed circuit breathing |

1070cc

| 9F/Sa/H | Engine numbers 26501 to 33660 and 19201 to 20410 |
| --- | --- |
| 9FD/Sa/H | Engine numbers 33661 to 33948 with closed circuit breathing and oil light switch in oil filter head |

1275cc

| 9F/Sa/Y | Engine numbers 31001 to 31504 pre closed circuit breathing with Purolator oil filter |
| --- | --- |
| 9FD/Sa/Y | Closed circuit breathing with oil light switch on filter housing after engine number |
|  | 32178 |
| 9FE/Sa/Y | Closed circuit breathing with oil light switch on Purolator filter housing and piston |
|  | design change |
| 9F/Sa/Y | From engine number 32378 onwards, closed circuit breathing with Purolator filter with oil light switch, Aeg510 camshaft introduced at engine number 40006, cross- drilled EN40Bcrank at engine number 42730 and metal dipstick tube at engine number 42548 onwards |
| 12H397 | Cooper S Mk3 with dynamo, negative earth and Tuftrided EN16T crankshaft |
| 12H398 | Cooper S Mk3 with alternator, negative earth and Tuftrided EN16T crankshaft |

Note: Only Cooper S engines with prefix '9F' had EN40B forged steel, nitrided crankshafts. All other A-series engines had EN16T steel crankshafts which were only Tuftrided on Mk3 'S' and Austin 1300GT (ADO16) and were stamped 12G1683.

1275cc, Solid wall block (no tappet chest covers)

| 12G/Ta/H | ADO17 shape Wolseley, Riley, MG & Vanden Plas fitted with S rods |
| --- | --- |
| 12H379 | 1275GT with dynamo, negative earth, electric fuel pump and remote type |
|  | gearbox |
| 12H380 | 1275GT with alternator, negative earth, electric fuel pump and remote type |
|  | gearbox |
| 12H389 | 1275GT with dynamo, negative earth, mechanical fuel pump and remote type |
|  | gearbox |
| 12H390 type | 1275GT with alternator, negative earth, mechanical fuel pump and remote |
|  | gearbox |
| 12H706 change | 1275GT with alternator, negative earth, mechanical fuel pump and rod |
|  | gearbox |
| 12H397 | Cooper S Mk3 with dynamo and negative earth |
| 12H398 | Cooper S Mk3 with alternator and negative earth |
| 12H/-/ | 1275 Austin ADO16 & Allegro |
| 12H610/635 | Innocenti Mini with duplex cam drive gears and 11-stud head |
| 12H719/832 | Innocenti Mini with simplex cam drive gears and 11-stud head |

1275cc, 1990 onwards (12A numbers)

| 12A/2A | Carburettor type |
| --- | --- |
| 12A/2B | Carburettor and catalyst type |
| 12A/2D | SPi (Single Point injection - Tbi, throttle body injection) with 9.4-1 CR |
| 12A/E | SPi (Tbi - throttle body injection) with 10-1 CR |

| With the following suffixes: |  |
| --- | --- |
| F53 | 10.0-1CR, 3.105 final drive, catalyst, oil cooler |
| F75 | 9.4-1CR, SPi, 3.105 final drive, closed loop catalyst |
| F76 | 9.4-1CR, automatic with closed loop catalyst |
| F77 | 10.0-1CR, SPi Cooper, 3.2 final drive, closed loop catalyst |
| G01 | 10.0-1CR, 3.105 final drive, catalyst, no oil cooler |
| G03 | 9.4-1CR, 3.105 final drive, catalyst, May 1992 onwards |
| G04 | 9.4CR, 2.76 final drive, catalyst, French only spec |
| G05 | 9.4-1CR, automatic with catalyst |

Factory replacement engine reference numbers (i.e. New, and Gold and Silver Seal units)

| 8G28 | 850cc - Oil feed to primary gear type crank with 1.375-inch tail shaft |
| --- | --- |
| 8G35 | 850cc - Deva bush conversion type crankshaft with 1.375-inch tail shaft |
| 8G45 | 850cc - Gold Seal, 1.5-inch self oiling primary gear/crankshaft type |
| GSE1101E | 850cc - Gold Seal unit |
| RKM1101E | 850cc - Silver Seal unit |
| BHM1307E | 850cc - Gold Seal unit, 85H prefix replacement |
| 8G18 | 997cc Cooper - Oil feed to primary gear type crankshaft |
| 8G29 | 997cc Cooper - Deva bush type primary gear and crankshaft |
| 8G36 | 997cc Cooper - Oil feed converted primary gear and crankshaft |
| 8G33 | 998cc Cooper - Fitted with 28G193 (12G202) cylinder head |
| 8G40 | 998cc Cooper - 28G222 (12G295)cylinder head introduced |
| 8G42 | 998cc Cooper - Closed circuit breathing rocker cover and side plates |
| 8G49 | 998cc Cooper - Purolator oil filter with oil light switch introduced |
| GSE1103E | 998cc Cooper - Gold Seal unit, was RKM1120E Silver Seal |
| RKM1103E | 998cc Cooper - Silver Seal unit |
| 8G57 | 998cc - New from 1969 and exchange for all pre-A+ up to 1981 |
| GSE1102E | 998cc -Gold Seal unit, was RKM1119E Silver Seal unit, pre-A+ up to 1981 |
| RKM1102E | 998cc - Silver Seal unit, pre-A+ up to 1981 |
| RKM1119E | 998cc - Silver Seal unit, replaced by RKM1102E, pre-A+ up to 1981 |
| BHM1232 | 998cc - Low compression dished piston, CAM6267 camshaft, early cylinder head and crankshaft main bearings with side locator tang, A+ |
| BHM1377 | 998cc - High compression flat top piston, CAM4717 camshaft, early cylinder head and crankshaft main bearings with side locator tang, A+ |
| BHM1458 | 998cc - Low compression piston, CAM6267 camshaft, late A+ cylinder head and crankshaft main bearings with central locator tang, A+ |
| BHM1460 | 998cc - High compression press-fit wrist pin piston, CAM4717 camshaft, late A+ cylinder head and crankshaft main bearings with central locator tang, A+ |
| LBB10089 | 998cc - Low compression press-fit wrist pin piston, CAM6267 camshaft, lead -free late A+ cylinder head and crankshaft main bearings with central locator tang, A+ |
| 8G156E | 1100cc - Clubman, Special and Austin 1100 (ADO16) |
| RKM1150E | 1100cc - Silver Seal unit for Clubman and Special |
| RKM1151E | 1100cc - Silver Seal unit for Clubman and Special |
| BHM1042 | 1100cc - New and Gold Seal units for Clubman and Special, 1974-1980 |
| BHM1229 | 1100cc - New and Gold Seal units for Clubman and Special, A+ Block, 1980 on |
| 8G38 | 970cc - Gold Seal unit, pre closed circuit breathing, Cooper S |
| 8G41 | 970cc - Gold Seal unit with closed circuit breathing sideplates, Cooper S |
| 8G46 | 970cc - Gold Seal unit with oil light switch in filter housing, Cooper S |
| 8G133 | 1070cc - Gold Seal unit, pre closed circuit breathing, Cooper S |
| 8G153 | 1070cc - Gold Seal unit with closed circuit breathing side plates, Cooper S |
| 8G145 | 1275cc - Pre closed circuit breathing unit, Cooper S |
| 8G154 | 1275cc - Closed circuit breathing side plates unit, Cooper S |
| 8G155 | 1275cc - Oil filter housing with oil pressure light switch, Cooper S |
| 8G174 | 1275cc - Fitted with AEG510 camshaft, Cooper S |
| 38G464 | 1275cc - Introduced in 1970, most of which were fitted with EN16 Tuftrided crankshaft |
| 8G200 | 1275cc - New and Gold Seal units Clubman 1275GT |
| 38G527 | 1275cc - New and Gold Seal units for engine numbers prefixed 12H706, 12H707 & 12H831 for Clubman 1275GT |
| RKM1130E | 1275cc - Silver seal unit, was RKM1112E, for Clubman 1275GT, pre-A+ |
| RKM1152E | 1275cc - Silver Seal unit for Clubman 1275GT, pre-A+ |
| RKM1112E | 1275cc - Silver Seal unit for Clubman 1275GT, pre-A+ |
| BHM1220 | 1275cc - A+ unit for Clubman 1275GT |
| 8G199E | 1275cc - Gold Seal unit with Duplex cam drive gears for 1300GT (ADO16) and |
|  | Innocenti 1300, 11-stud cylinder head |
| 38G559E | 1275cc - Gold Seal unit with Simplex cam drive gears for engine numbers prefixed 12H719 and 12H832, for 1300GT (ADO16) and Innocenti 1300, 11- stud cylinder head |
| GSE1109E | 1275cc - Gold Seal unit for 1300GT (ADO16) and Innocenti 1300, 11-stud cylinder head |
| RKM1133E | 1275cc - Silver Seal unit for 1300GT (ADO16) and Innocenti 1300, 11-stud cylinder head |
| BHM1209E | 1275cc - Gold Seal unit, replaced all Innocenti 1300 engines from engine numbers prefixed 12H610, 12H635 and 12H832 |

Notes:

All engines pre-fixed with 'GSE' or 'RKM' were of UNIPART origin when BL split its parts organisation.

'E' at the end of a part number denotes it is a reconditioned, service unit only (basic engine assembly with no ancillaries).

'N' at the end of a part number denotes a brand new unit.

Engine - Metro engine identification data

The widespread and prolific fitment of Metro engines into Minis - usually those endowed as standard with small-bore (850/998/1098) engines - practiced in the UK has been spreading to many other countries worldwide. So it's not surprising that the most commonly asked question has also become more prolific concerning the fitment of said engine - apart from actually how to manage the transplant and the differences in the installations, covered in articles relevant article on this site - is just which type of engine the individual concerned has bought or what they already have fitted to their car and is it worth keeping or junking. One initial question I pose often draws a pregnant silence whilst said inquire scrambles their brains trying to find an answer for. Following that protracted pause, 'I dunno' is the usual answer. I then have to get into a lengthy explanation as to how to determine what said engine is. To circumvent as many of these instances as possible, and endow all and sundry with the wherewithal to do the detective work for themselves, I have here produced the most comprehensive listing of engine identification data for the Metro A+ engines I could unearth given a reasonable amount of time. There should be enough here for everybody to make a very fair identification of what they have and what they're looking for. The number you will need is that stamped into an aluminium plate riveted to the engine block just below the thermostat housing.

A word of warning - whilst the data given here is as accurate and complete as I could make it without 6 months-worth of hermit-like investigation, there are bound to be oddities. Austin/Morris, BL/Austin Rover Group/Rover were never dedicated at keeping 100% correct records, particularly post 1970 - that's why Heritage really struggle with trying to identify cars manufactured after 1969. The Metro data - or lack of it - proves the point. It's nothing like as comprehensive as the Mini stuff.

Whilst most of the engines do not really give a whole lot of difference in performance, the one outstanding version is the MG Metro engine. Where the majority of the variously similar power plants put out around 63-65bhp, the MG metro achieves a more spirited 70-74bhp. This is due to a higher compression ratio (10 -1), larger inlet valve (35.7mm instead of 33.3mm), and - mainly - a sportier camshaft with half-decent induction and exhaust systems. The inlet manifold being a reasonably good flowing, water heated aluminium example. Rover finally got the hint having witnessed the prolific after-market fitment or such an item to Minis in epic proportions for years. The exhaust manifold is a cast iron 'LCB' style with a pair of exhaust downpipes instead of the more common single item and works very efficiently. A bonus of this over the more common after-market steel-tubed LCBs is that it is much quieter by dint of absorbing more exhaust 'noise'. Coupled to a pair of Maniflow tubular steel downpipes it is very nearly as efficient as a full tubular steel manifold. The cylinder head is more along the lines of the old Cooper S spec and in fact flows slightly more air than it's predecessor, the camshaft being the sportiest fitted to any production A-series engine. It uses the inlet profile and timing of the original 997 Cooper cam and the exhaust specification of the old Leyland Special Tuning '731' fast road cam.

1st prefix group identifies engine capacity -

85 = 850cc (mainly used in export only models)

99 = 998cc

12 = 1275cc

1st prefix letter identifies engine orientation -

H = Transverse

2nd prefix group identifies gearbox, ancillary and specific engine type -

prefix number identifies general engine installation details (i.e. '907')

prefix letter identifies emissions equipment type (i.e. 'P')

3rd prefix group identifies compression ratio and serial number

prefix letter denotes compression ratio -

H = High compression

L = Low compression

Prefix number is engine serial number (i.e. 101, 102, etc.)

2nd prefix number combination references

| 897 | Yd + Bf + Hm + Zj +Sk |
| --- | --- |
| 898 | Yd + Bf + Hm + Zj + Tj |
| 907 | Yd + Bf + Hm + Sk |
| 908 | Yd + Bf + Hm + Tj |
| 929 | Yd + Bf + Hm + Dn + Sk |
| 951 | Yd + Bf + Hm + Zj + Dn + Sk |
| 955 | Yd + Bf + Hm + Nl + Sk |
| 958 | Yd + Bf + Hm + Nn + Sk |
| 959 | Gd + Bf + Hm + Fp + Sk |
| 962 | Yd + Bf + Hm + Hf + Sk |
| 963 | Yd + Bf + Hm + Hf + Zj + Sk |
| 977 | Yd + Bf + Hm + Xc + Jn + Sk |
| 978 | Yd + Bf + Hm + Xc + Dn + Sk |
| 996 | Yd + Bf + Yn + Sk |
| A02 | Yd + Bf + Hm + Nn + Sk |
| A05 | Yd + Bf + Hm + Hf + Zj + Kp + Sk |
| A06 | Yd + Bf + Hm + Kp + Sk |
| A07 | Yd + Bf + Hm + Zj + Lp + Sk |
| A08 | Yd + Bf + Hm + Lp + Sk |
| A09 | Yd + Bf + Hm + Nl + Lp + Sk |
| A47 | Yd + Bf + Hm + Vn + Sk |

2nd prefix letter reference details

| Bf | Alternator with negative earth |
| --- | --- |
| Dn | Base line engine with single outlet exhaust manifold |
| Fp | Metro high performance derivative (meant to identify high compression, sportier than original and initial standard cam and can include MG Metro specification derivatives) |
| Gp | High pressure electric fuel pump (therefore MG Metro Turbo) |
| Hf | 1st alternative compression ratio (should be 9.71CR as used in early pre 1983 range) |
| Hm | LC8 design variant (probably means the 'Low Compression' A+ derivative of the A series engine as in '&' denotes a Mini engine) |
| Jn | 1980 requirements (probably closed circuit breathing and lean running) |
| Kp | 3.65 Final Drive fitted to gearbox |
| Lp | 3.105 Final Drive fitted to gearbox |
| Nl | Hot climate specification (usually more conservative ignition advance curve in dizzy, different thermostat and different fuelling, i.e. carb needle) |
| Nn | Alternative compression ratio (should be 9.751CR as used in post 1983 general range) |
| Sk | Standard ratio gearbox with 3.44 Final Drive, inboard CV joints and rod change |
| Tj | Automatic gearbox fitment |
| Vr | Post Office spec i.e. low compression and low final drive |
| Yc | Police spec i.e. high compression, probably MG cam and up rated diff pin |
| Yd | Mechanical fuel pump |
| Yn | MG Metro design/specification variants (high CR, sportier CAM6648 cam, bigger inlet valve in head) also known to be fitted to LHD Van Den Plas |
| Zj | Cold country spec (more advanced advance curve in dizzy, different thermostat and so on) |

2nd prefix letter data

| AA | Carburettor crankcase ventilation and intake air temperature control |
| --- | --- |
| P | Carburettor crankcase and clean air package |

Example;  99H907PH101

| 99 | 998cc |  |  |
| --- | --- | --- | --- |
| H | Transverse |  |  |
| 907 | Yd + Bf + Hm + Sk |  |  |
|  |  | Yd | Mechanical fuel pump |
|  |  | Bf | Alternator with negative earth |
|  |  | Hm | LC8 variant A+ Metro engine |
|  |  | Sk | Standard gearbox ratios CV joints, rod change, inboard CV joints, rod change |
| P | Carburettor crankcase and clean air package |  |  |
| H | High compression |  |  |
| 101 | Engine serial number |  |  |